Shuster Boring Investigation

March 26 – April 3, 2012

	Soil Boring
	Sample

Depth
	Weight of Foil
	Weight of Foil & Soil
	Weight of Soil


	Blow Counts
	Soil Description

	SB 1
	9-11
	4
	184
	180
	10-3-9-15
	Loose pale gray fine-med sand wet SP

	
	19-21
	5
	153
	148
	50-39-50-58
	Dense pale gray fine-med. Sand trace fine gravel wet SP

	
	29-31
	4
	160
	156
	18-28-18-36
	Medium dense fine-med. Sand wet SP

	
	34-36
	4
	168
	164
	21-24-51-34
	Medium dense yellow brown medium-coarse Sand little silt trace coarse fine medium gravel wet SM

	SB 2
	0.5-2
	7
	167
	160
	9-9-12
	Loose pale brown fine-medium Sand damp SP

	
	9-11
	4
	154
	150
	8-8-11-8
	Loose pale brown fine-coarse Sand wet SW

	
	14-16
	6
	165
	159
	29-25-34-31
	Med dense pale gray fine-med Sand trace medium-coarse gravel wet SW

	
	29-31
	5
	158
	153
	36-34-120/1”
	Med dense pale brown medium coarse Sand trace fine gravel wet SW

	SB 3
	4-6
	5
	214
	209
	24-29-120/1”
	Med dense fine to med Sand wet SP

	
	9-11
	7
	167
	160
	6-3-4-5
	Loose dark gray fine-medium Sand wet SP

	
	14-16
	8
	206
	198
	40-41-33-38
	Dense pale brown fine-medium Sand wet SP

	
	19-21
	10
	240
	230
	60-35-32-30
	Very dense fine-medium Sand trace fine gravel wet SP

	
	24-26
	8
	241
	233
	34-28-45-35
	Dense fine-coarse Sand trace fine gravel wet SW

	

	34-36
	4
	160
	156
	67-120/3”
	Very dense medium-coarse Sand and little med-coarse gravel  wet SW

	SB 4
	0.5-2
	6
	184
	178
	9-14-22
	Loose pale yellow fine-med. Sand damp SP

	
	9-11
	4
	184
	180
	3-4-12-13
	Loose pale gray fine-med. Sand trace fine gravel wet SP



	
	14-16
	6
	157
	151
	29-25-26-22
	Pale gray fine-med Sand little fine gravel wet SW

	
	24-26
	4
	168
	164
	44-37-40-36
	Med dense pale gray fine –coarse Sand little fine gravel wet SW

	
	29-31
	8
	221
	213
	44-40-22-15
	Med dense gray fine-med. Sand little fine-med gravel wet

	
	34-36
	8
	202
	194
	54-87-53-34
	Dense deep yellow brown fine-med Sand trace fine gravel little silt wet SP-SM

	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	


Notes: 

Weight for all foil and soil in grams

All samples collected from undisturbed split spoon

Undisturbed sample approximate size 1.5”D x 3”L 

Sample intervals in feet

Borings where first sample interval is 0.5’-2.0’ accounts for  0.5’ of asphalt pavement

Blow counts are per 6-inch intervals
